ThermaKlip

SECTION 07 42 13

THERMAL ISOLATION CLIPS FOR EXTERIOR WALL PANEL ASSEMBLIES

PART 1 – GENERAL

1.1 SECTION INCLUDES

ThermaKlip – Stainless Steel Thermal Isolation Clip for exterior wall assemblies.

- Clip mounting fasteners as per project engineer’s recommendations.

1.2 RELATED SECTIONS

- Section 05 10 00 – Structural Metal Framing.

- Section 05 40 00 – Cold-Formed Metal Framing.

- Section 07 21 00 – Thermal Insulation.

- Section 06 10 00 – Rough Carpentry.

1.3 REFERENCES

- ASHRAE 90.1 – Energy Standard for Buildings Except Low-Rise Residential Buildings.

- ASTM A240 – Standard Specification for Chromium and Chromium-Nickel Stainless Steel Plate, Sheet, and Strip for Pressure Vessels and for General Applications.

- ASTM C1363 – Standard Test Method for Thermal Performance of Building Materials and Envelope Assemblies by Means of a Hot Box Apparatus.

- CAN/ULC-S134 – Standard Method of Fire Test of Exterior Wall Assemblies.

- NFPA 285 – Standard Fire Test Method for Evaluation of Fire Propagation Characteristics of Exterior Non-Load-Bearing Wall Assemblies Containing Combustible Components.

- National Energy Code for Buildings (NECB) SB-10 – Supplementary Standard.

1.4 SUBMITTALS

- Product Data: Manufacturer’s specifications, preparation requirements, storage, handling, and installation recommendations.

- Shop Drawings: Include product dimensions, details of materials, construction, finish, and integration with adjacent components.

- Samples: Provide two representative units of each type and size.

- Test Reports: Thermal and fire performance reports demonstrating compliance with the specified standards.

1.5 QUALITY ASSURANCE

- Manufacturer Qualifications: Manufacturer must have a minimum of five years of experience in producing stainless steel thermal clips for exterior wall assemblies.

- Installer Qualifications: Installer must have a minimum of two years of experience with similar projects.

- Mock-Up: Construct a mock-up for evaluation of workmanship and appearance, retaining it as a reference for construction.

** NOTE TO SPECIFIER **  Include mock-up if the project size or quality warrant the expense. The following is one example of how a mock-up on might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

1.6 DELIVERY, STORAGE, AND HANDLING

- Store and handle ThermaKlip in accordance with manufacturer’s instructions to prevent damage.

- Protect from weather exposure and mechanical damage.

1.7 WARRANTY

- Manufacturer’s standard limited warranty.

PART 2 – PRODUCTS

2.1 MANUFACTURER

- Acceptable Manufacturer: [Insert Manufacturer’s Name and Contact Information]

- Substitutions are not permitted without prior approval.

2.2 PERFORMANCE AND DESIGN REQUIREMENTS

- Material: 14-gauge ASTM A240 S304 Stainless Steel.

- High Load Capacity: Fewer clips required compared to synthetic or aluminum clips, reducing thermal bridging.

- Thermal Performance: Modeled and evaluated in accordance with ASTM C1363.

- Fire Resistance: Complies with CAN/ULC-S134 and NFPA 285.

- Versatile Installation: Compatible with concrete, concrete block, steel studs, and wood substrates.

- Effective R-Value Contribution: Determined in combination with the insulation system specified.

- Mounting Orientation: Suitable for both vertical and horizontal sub-girt applications.

2.3 AVAILABLE SIZES

ThermaKlip is available in multiple sizes to accommodate varying insulation thicknesses:

- 2.0-inch Clip: Supports insulation thickness from 2" to 3.5" (51 to 89 mm) and allows for ±1/4-inch (6 mm) deviation.

- 3.25-inch Clip: Supports insulation thickness from 4" to 6" (101 to 152 mm) and allows for ±1/2-inch (13 mm) deviation.

- 4.75-inch Clip: Supports insulation thickness from 5" to 8" (127 to 203 mm) and allows for ±1/2-inch (13 mm) deviation.

PART 3 – EXECUTION

3.1 EXAMINATION

- Verify that substrates are ready to receive ThermaKlip installation.

- Report any discrepancies or unsatisfactory conditions before proceeding.

3.2 PREPARATION

- Clean all surfaces before installation.

- Ensure substrate is level and structurally sound.

3.3 INSTALLATION

- Install ThermaKlip in accordance with manufacturer’s written instructions and approved shop drawings.

- Ensure clips are plumb and aligned, maintaining a tolerance of ±1/8 inch over 20 feet (3.2 mm over 6.1 m).

- Secure clips with recommended fasteners, following specified spacing for structural load requirements.

- Install L-girt after clips are in place, ensuring proper attachment and alignment.

3.4 FIELD QUALITY CONTROL

- Coordinate field inspections per Division 01 requirements.

- Ensure compliance with manufacturer’s installation recommendations.

3.5 CLEANING AND PROTECTION

- Clean installed components as per manufacturer’s instructions.

- Replace any damaged components prior to project completion.

END OF SECTION